READ.ME

Project1d.py

- Imports numpy, matplotlib, and SciPy tools for numerical solving and root-finding.

- Defines a small spherical silica particle:

  - Radius: 5 µm

  - Density: 2200 kg/m^3

  - Calculates its volume, mass, and weight mg.

- Defines gas/air properties at low pressure:

  - Pressure: 30 Pa

  - Temperature: 293 K

  - Calculates gas density, mean free path, and Knudsen number.

- Defines laser parameters:

  - Wavelength: 532 nm

  - Beam waist: 3.5 µm

  - Laser power: 0.1 W

  - Calculates Gaussian peak intensity and Rayleigh length.

- Models three phenomenological upward/vertical forces on the particle:

  - Scattering/radiation pressure force

  - Gradient force

  - Photophoretic force caused by laser heating in gas

- Combines those forces into a total optical force:

- F_total = F_scat + F_grad + F_photo

- Calculates the net force by subtracting gravity:

- F_net = F_total - mg

- Searches for equilibrium positions where the upward forces balance gravity.

- Checks which equilibrium points are stable using the slope condition:

- dF_net/dz < 0

- Chooses the first stable equilibrium position.

- Linearises the nonlinear force near equilibrium to estimate:

  - Effective spring constant k

  - Oscillation angular frequency omega

  - Oscillation frequency in Hz

- Calculates gas damping using different drag models:

  - Stokes drag

  - Cunningham-corrected Stokes drag

  - Epstein drag

  - Or automatically chooses one based on Knudsen number

- Simulates the particle’s motion over time using two models:

  - A full nonlinear force model

  - A linear simple harmonic motion approximation

- Starts the particle slightly displaced from equilibrium by 50 µm.

- Solves both motion equations using solve_ivp.

- Plots:

  - Nonlinear motion vs linear SHM approximation

  - Individual force curves and total force

  - Nonlinear net force vs linearised net force near equilibrium

  - Linear and log fourier spectums

Project_1dnoise.py

- Imports numerical, plotting, ODE-solving, root-finding, and spectral-analysis tools:

- numpy

- matplotlib

- solve_ivp

- brentq

- welch

 Defines a spherical silica particle:

- Radius: 10 µm

- Density: 2200 kg/m^3

- Calculates volume, mass, and bare weight.

  Defines air/gas conditions:

- Pressure: 30 Pa

- Temperature: 300 K

- Calculates gas density, mean free path, and Knudsen number.

  Includes buoyancy:

- Calculates the upward buoyancy force from displaced gas.

- Uses an effective downward weight:

F_weight_effective = F_weight - F_buoyancy

  Defines laser parameters:

- Wavelength: 532 nm

- Beam waist: 3.5 µm

- Power: 0.1 W

- Calculates Rayleigh length and peak Gaussian intensity.

  Models the laser intensity along the vertical z direction, with maximum intensity at the focus.

  Defines three optical/thermal forces:

- Scattering force, upward along the beam.

- Gradient force, pulling the particle toward the focus.

- Photophoretic force, caused by laser heating of the particle in gas.

  Combines the forces into a total upward force:

F_total = F_scat + F_grad + F_photo

  Defines the net force by subtracting effective weight:

F_net = F_total - F_weight_effective

  Searches over z from -10 mm to +10 mm to find equilibrium points where:

F_net = 0

  Uses brentq root-finding to accurately locate each equilibrium.

  Checks stability using:

dF_net/dz < 0

meaning a small displacement produces a restoring force.

  Chooses the first stable equilibrium position.

  Linearises the force near the stable equilibrium to calculate:

- Effective spring constant k

- Angular frequency omega

- Oscillation frequency in Hz

- Expected thermal RMS displacement

  Defines several damping models:

- Stokes drag

- Cunningham-corrected Stokes drag

- Epstein drag

- Automatic selection based on Knudsen number

  Calculates the selected damping coefficient b and damping ratio.

  Solves two deterministic motion models:

- Full nonlinear force model

- Linear simple harmonic oscillator approximation

  Starts the particle slightly displaced from equilibrium by 20 µm.

  Adds a stochastic Brownian dynamics simulation using the BAOAB Langevin method:

- Includes deterministic optical forces

- Includes gas damping

- Includes random thermal velocity kicks

- Uses a fixed random seed for repeatability

  Compares the simulated Brownian RMS displacement with the theoretical thermal RMS displacement.

  Plots a histogram of Brownian position fluctuations around equilibrium.

  Compares that histogram with the expected Gaussian thermal distribution.

  Plots the same histogram on a semilog scale to check the distribution tails.

  Plots time-domain motion:

- Nonlinear deterministic trajectory

- Brownian BAOAB trajectory

- Stable equilibrium line

  Includes, but comments out, plots of the individual force curves and net-force linearisation.

  Computes Fourier transforms of the motion to identify oscillation frequencies.

  Uses Welch’s method to calculate power spectral densities, in units of displacement PSD.

  Plots PSDs for:

- Deterministic nonlinear motion

- Full Brownian trajectory

- Early Brownian transient motion

- Late-time Brownian steady-state motion

  Compares the late-time Brownian PSD with the theoretical damped harmonic oscillator thermal PSD

Project_1d_lasernoise

  imports tools for:

- numerical arrays with numpy

- plotting with matplotlib

- solving ODEs with solve_ivp

- finding equilibrium roots with brentq

- calculating PSDs with welch

  Defines two noise switches:

USE_BROWNIAN_NOISE = True

USE_LASER_POWER_NOISE = True

  Defines a silica particle:

- Radius: 10 µm

- Density: 2200 kg/m^3

- Calculates volume, mass, and weight.

  Defines gas conditions:

- Pressure: 30 Pa

- Temperature: 130 K

- Calculates gas density, buoyancy, effective weight, mean free path, and Knudsen number.

  Defines laser parameters:

- Wavelength: 532 nm

- Beam waist: 3.5 µm

- Power: 0.1 W

- Calculates Rayleigh length and peak Gaussian intensity.

  Defines a 1D Gaussian-like axial intensity profile with maximum at the laser focus.

  Models three upward/vertical forces:

- Scattering/radiation-pressure force

- Axial gradient force

- Photophoretic force

  The scattering and gradient forces are phenomenological:

- Q_pr = 0.1 sets the scattering strength.

- Fg0 = 5.0 * F_weight sets the gradient-force strength.

  The photophoretic force uses a Rohatschek-style pressure-dependent model.

  Combines the forces into:

F_total = F_scat + F_grad + F_photo

  Defines the net force as:

F_net = F_total - F_weight_effective

  Searches from -10 mm to +10 mm to find equilibrium positions where F_net = 0.

  Uses the slope condition:

dF_net/dz < 0

to select stable equilibria.

  Linearises the force near the stable equilibrium to calculate:

- Effective spring constant k

- Angular trap frequency omega

- Oscillation frequency in Hz

- Expected thermal RMS displacement

  Calculates gas damping using:

- Stokes drag

- Cunningham-corrected Stokes drag

- Epstein drag

- Automatic drag-model selection from the Knudsen number

  Defines deterministic equations of motion:

- A nonlinear model using the full force

- A linear simple-harmonic approximation

  Starts the particle displaced from equilibrium by:

displacement = 50e-6

  Sets up a simulation time of 1 s.

  Uses an experimental sampling frequency of 20 kHz.

  Oversamples the simulation by a factor of 10, giving a simulation timestep of:

dt_simulation = 5e-6 s

  Solves both deterministic models using solve_ivp.

  Implements a BAOAB Langevin integrator for stochastic dynamics.

  Adds optional stepwise laser-power noise:

- Laser power varies between 0.99 P, 1.00 P, and 1.01 P.

- The step duration is about 0.01 s.

- The power changes only between neighbouring allowed levels.

  Creates different BAOAB trajectories:

- Baseline: no Brownian noise and no laser-power noise

- Brownian-only trajectory, if enabled

- Laser-noise-only trajectory, if enabled

- Main trajectory using whichever noise switches are active

  plots Brownian histogram and late-time Brownian PSD plots

  Since laser-power noise is on, it plots:

- Laser power versus time

- Isolated displacement caused by laser-power noise

  Plots the nonlinear deterministic trajectory against the main BAOAB trajectory.

  Includes commented-out force-curve and net-force plots.

  Computes Fourier/PSD information for the deterministic nonlinear motion.

  Uses both:

- Welch-averaged PSD

- Raw periodogram PSD

  Plots PSDs with markers for:

- Fundamental trap frequency

- Second harmonic

- Third harmonic

- Nyquist limit

  Computes and plots PSDs for:

- Nonlinear deterministic motion

- Full BAOAB trajectory with the selected noise sources

- Early-time BAOAB transient motion

  If Brownian noise is enabled, it also compares the late-time Brownian PSD with the theoretical damped harmonic oscillator PSD.

Project_1d_noise_feedback.py

- Imports tools for numerical modelling, plotting, ODE solving, root finding, and PSD analysis.

- Simulates 1D vertical optical levitation of a silica microsphere.

- Uses these active settings:

- USE_BROWNIAN_NOISE = True

- USE_LASER_POWER_NOISE = True

USE_PD_FEEDBACK = True

So the main simulation includes Brownian motion, laser-power noise, and feedback control.

- Defines feedback-control settings:

  - Position measurement noise is turned off

  - Loop delay: 50 µs

  - Proportional gain based on trap stiffness

  - Derivative gain based on gas damping

  - Laser power limited between 80% and 120% of nominal power

- Defines the particle:

  - Radius: 10 µm

  - Density: 2200 kg/m^3

  - Calculates volume, mass, and gravitational weight.

- Defines gas properties:

  - Pressure: 30 Pa

  - Temperature: 300 K

  - Calculates gas density, buoyancy, effective weight, mean free path, and Knudsen number.

- Defines laser properties:

  - Wavelength: 532 nm

  - Beam waist: 3.5 µm

  - Power: 0.1 W

  - Calculates Rayleigh length and peak intensity.

- Defines a 1D axial intensity profile with maximum at the laser focus.

- Models three vertical force contributions:

  - Scattering/radiation-pressure force

  - Gradient force

  - Photophoretic force

- Combines the optical/photophoretic forces into:

F_total = F_scat + F_grad + F_photo

- Defines the net force as:

F_net = F_total - F_weight_effective

- Searches for equilibrium positions where the upward force balances effective weight.

- Selects a stable equilibrium using:

dF_net/dz < 0

- Linearises the force around the stable equilibrium to calculate:

  - Effective spring constant k

  - Trap angular frequency omega

  - Trap frequency in Hz

  - Expected thermal RMS displacement

- Calculates gas damping using:

  - Stokes drag

  - Cunningham-corrected Stokes drag

  - Epstein drag

  - Automatic regime selection based on Knudsen number

- Defines two deterministic motion models:

  - Full nonlinear force model

  - Linear simple-harmonic approximation

- Starts the particle 50 µm away from the equilibrium position:

displacement = 50e-6

- Sets simulation timing:

  - Experimental sampling frequency: 20 kHz

  - Simulation oversampling factor: 10

  - Simulation timestep: 5 µs

  - Total duration: 1 s

- Solves the deterministic nonlinear and linear models using solve_ivp.

- Implements a BAOAB Langevin integrator for stochastic Brownian dynamics.

- Generates Brownian random velocity kicks using a fixed random seed.

- Adds stepwise laser-power noise:

  - Power varies by ±1%

  - Step duration is 0.01 s

  - Power switches between 0.99, 1.00, and 1.01 times nominal power

- Runs several BAOAB trajectories:

  - Baseline with no noise

  - Brownian-only trajectory

  - Laser-noise-only trajectory

  - Main Brownian plus laser-noise trajectory

  - Brownian plus laser-noise trajectory with PD feedback

- Implements a PD feedback loop:

  - Measures delayed particle position

  - Optionally adds measurement noise, currently off

  - Filters the measured position

  - Estimates velocity from filtered position

  - Computes a feedback force

  - Converts that force into a laser-power correction

  - Clips the laser command within allowed limits

- Prints diagnostic values such as:

  - Brownian RMS displacement

  - Laser-noise RMS displacement

  - Feedback RMS displacement

  - Feedback gain values

  - Actual loop delay

  - Laser command range

- Plots laser power versus time.

- Plots feedback laser power versus time.

- Plots isolated laser-noise displacement.

- Plots isolated Brownian displacement.

- Plots a Brownian position histogram and compares it with the expected thermal Gaussian distribution.

- Plots nonlinear BAOAB motion and BAOAB motion with feedback.

- Plots trajectories with and without feedback.

- Computes Fourier and PSD information using:

  - FFT-style raw periodograms

  - Welch-averaged PSDs

- Plots PSDs for:

  - Deterministic nonlinear motion

  - Full noisy BAOAB motion

  - BAOAB motion with feedback

  - Early-time Brownian/noisy motion

  - Late-time Brownian motion

- Compares the late-time Brownian PSD with the theoretical damped harmonic oscillator PSD.

Project_2d.py

  Imports tools for numerical modelling, plotting, ODE solving, and root finding.

  Simulates 2D optical levitation of a silica microsphere in the x-z plane.

  Defines the particle:

- Radius: 5 µm

- Density: 2200 kg/m^3

- Calculates volume, mass, and gravitational weight.

  Defines gas properties:

- Pressure: 30 Pa

- Temperature: 293 K

- Calculates gas density, mean free path, and Knudsen number.

  Defines laser/beam parameters:

- Beam waist: 20 µm

- Axial length scale: 100 µm

- Laser power: 0.1 W

  Defines force amplitudes relative to particle weight:

- Scattering force strength: 2.5 mg

- Gradient force strength: 1.0 mg

  Defines a 2D Gaussian-like beam intensity:

intensity(x, z)

  Calculates the intensity gradients:

dI/dx

dI/dz

  Uses those gradients to define a 2D gradient force:

F_grad_2d(x, z)

  Defines a 2D scattering force:

- No transverse component

- Upward vertical component proportional to local intensity

  Defines a photophoretic force:

- Uses gas pressure, particle thermal conductivity, accommodation coefficient, and absorbed intensity

- Acts upward in the +z direction

- Has no transverse component in this model

  Combines the optical and photophoretic forces into:

F_optical_2d = F_grad_2d + F_scat_2d + F_photo_2d

  Defines the on-axis net vertical force:

Fz_net_on_axis = Fz - mg

  Searches for on-axis equilibrium positions where the vertical force balances gravity.

  Selects a stable equilibrium using:

dFz_net/dz < 0

  Sets the equilibrium position as:

x_eq = 0

z_eq = stable root

  Calculates local spring constants near equilibrium:

- kx from the transverse force slope

- kz from the vertical force slope

  Calculates linear trap frequencies:

- fx

- fz

  Calculates expected thermal RMS displacement in both directions:

- x_rms_thermal

- z_rms_thermal

  Calculates pressure-dependent gas damping using Cunningham-corrected Stokes drag.

  Prints damping ratios for both x and z.

  Defines a deterministic 2D nonlinear equation of motion:

- Position variables: x, z

- Velocity variables: vx, vz

- Includes optical forces, gravity, and drag

  Starts the particle displaced from equilibrium:

- x_displacement = 26 µm

- z_displacement = 80 µm

  Solves the deterministic 2D nonlinear motion using solve_ivp.

  Also simulates a Brownian/Langevin trajectory using an Euler-style update:

- Adds random thermal velocity kicks in both x and z

- Uses timestep dt = 1 µs

- Runs for 1 s

  Prints late-time simulated Brownian RMS displacement in x and z.

  Plots deterministic motion versus time:

- x(t)

- z(t)

  The Brownian time traces are present but currently commented out.

  Computes Fourier spectra of the deterministic x and z motion.

  Plots the normalized frequency spectra and marks:

- Linear fx

- Linear fz

  Plots the 2D deterministic trajectory in the x-z plane.

  The Brownian trajectory plot is present but currently commented out.

  Plots a 2D force-field map:

- Background shows normalized beam intensity

- White arrows show net force direction and relative strength

- Red point marks the equilibrium position

  Includes an optional on-axis force check section, currently commented out.

Project_2d_improved.py

  mports tools for numerical modelling, plotting, ODE solving, and root finding.

  Simulates 2D optical levitation of a particle in the x-z plane.

  Defines the particle:

- Radius: 5 µm

- Density: 1000 kg/m^3

- Refractive index: 1.46

- Surrounding medium refractive index: 1.00027

- Calculates volume, mass, and gravitational weight.

  Defines gas properties:

- Pressure: 3 Pa

- Temperature: 293 K

- Calculates gas density, mean free path, and Knudsen number.

  Defines laser/beam parameters:

- Beam waist: 20 µm

- Axial length scale: 100 µm

- Laser power: 0.08 W

- Peak Gaussian intensity

  Defines a 2D Gaussian-like beam intensity:

intensity(x, z)

  Defines the physical intensity:

physical_intensity = I0 * intensity

  Defines a Gaussian-beam wavefront radius of curvature:

wavefront_radius(z)

  Uses an Ashkin-style ray-optics force model instead of the older phenomenological gradient/scattering forces.

  Samples the projected disk of the particle using a 2D grid of ray bundles.

  For each ray bundle, calculates:

- Incident angle

- Refracted angle

- Fresnel reflection coefficient

- Fresnel transmission coefficient

- Scattering-like efficiency Q_s

- Gradient-like efficiency Q_g

  Uses the local beam direction to calculate ray-optics force components:

- Scattering-like force along the local ray direction

- Gradient-like force perpendicular to the local ray direction

  Splits the ray force into:

Fx_scat, Fz_scat, Fx_grad, Fz_grad

  Combines the ray-optics components into:

F_ray_optics_2d

  Defines a photophoretic force:

- Uses gas pressure, particle thermal conductivity, accommodation coefficient, and absorbed intensity

- Acts upward in the +z direction

- Has no transverse component in this model

  Combines the ray-optics and photophoretic forces into:

F_optical_2d = F_ray_optics_2d + F_photo_2d

  Defines the on-axis net vertical force:

Fz_net_on_axis = Fz - mg

  Searches for on-axis equilibrium positions over -10 mm to +10 mm.

  Selects stable equilibria using:

dFz_net/dz < 0

  Sets the equilibrium position as:

x_eq = 0

z_eq = stable root

  Calculates local spring constants near equilibrium:

- kx from the transverse force slope

- kz from the vertical force slope

  Checks that both kx and kz are positive.

  Calculates linear trap frequencies:

- fx

- fz

  Calculates expected thermal RMS displacement in both directions:

- x_rms_thermal

- z_rms_thermal

  Calculates gas damping using:

- Stokes drag

- Cunningham-corrected Stokes drag

- Epstein drag

- Automatic model choice based on Knudsen number

  Prints damping ratios for both x and z.

  Defines a deterministic 2D nonlinear equation of motion:

- Position variables: x, z

- Velocity variables: vx, vz

- Includes ray-optics force, photophoretic force, gravity, and drag

  Defines a linear 2D SHM approximation:

- Uses kx and kz

- Includes damping in both directions

  Starts the particle displaced from equilibrium:

- x_displacement = 6 µm

- z_displacement = 10 µm

  Prints the initial force and acceleration values.

  Solves the deterministic nonlinear 2D model using solve_ivp.

  Solves the linear 2D SHM approximation using solve_ivp.

  Extracts nonlinear and linear solutions for:

- x

- z

- vx

- vz

  Plots nonlinear x(t) and z(t) motion.

  Computes Fourier spectra of the nonlinear x and z motion.

  Plots separate log-log Fourier spectra for:

- x motion

- z motion

  Plots the 2D nonlinear trajectory in the x-z plane.

  Checks the transverse restoring force at z = z_eq.

  Warns if the chosen initial x displacement may be outside the local restoring region.

  Plots a vertical force breakdown along the beam axis:

- Ray scattering-like axial force

- Ray gradient-like axial force

- Total ray-optics axial force

- Photophoretic force

- Gravity

- Total net vertical force

  Plots a 2D force-field map:

- Background shows normalized beam intensity

- White arrows show net force direction and relative strength

- Red point marks the equilibrium position

  Includes an optional on-axis force check section, currently commented out.

Project_2d_noise.py

  mports tools for numerical modelling, plotting, interpolation, root finding, timing, and PSD analysis.

  Simulates 2D optical levitation of a particle in the x-z plane.

  Defines the particle:

- Radius: 5 µm

- Density: 1000 kg/m^3

- Particle refractive index: 1.46

- Medium refractive index: 1.00027

- Calculates volume, mass, and gravitational weight.

  Defines gas properties:

- Pressure: 3 Pa

- Temperature: 293 K

- Calculates gas density, mean free path, and Knudsen number.

  Defines laser/beam parameters:

- Beam waist: 20 µm

- Wavelength: 1064 nm

- Beam quality factor M2 = 1.2

- Laser power: 0.08 W

- Peak Gaussian intensity

  Allows the Rayleigh range to be either:

- Manually set to 100 µm

- Calculated from the M2 beam-quality relation

  Adds laser-power noise settings:

- Noise amplitude: ±1%

- Step duration: 0.01 s

  Defines a 2D Gaussian-like beam intensity:

intensity(x, z)

  Defines the physical intensity:

physical_intensity = power_factor * I0 * intensity

  Uses an Ashkin-style ray-optics force model.

  Samples the projected disk of the sphere using many ray bundles.

  For each ray bundle, calculates:

- Incident angle

- Refracted angle

- Fresnel reflection coefficient

- Fresnel transmission coefficient

- Scattering-like efficiency Q_s

- Gradient-like efficiency Q_g

  Computes ray-optics force components:

- Scattering-like force along the local ray direction

- Gradient-like force perpendicular to the local ray direction

  Defines a photophoretic force:

- Depends on absorbed intensity

- Acts upward in the +z direction

- Has no transverse component in this model

  Combines ray-optics and photophoretic forces into:

F_optical_2d

  Builds an optional force lookup table:

- Precomputes force over an x-z grid

- Uses interpolation during time integration

- Speeds up the BAOAB simulation

  Finds on-axis equilibrium positions where:

Fz - mg = 0

  Selects stable equilibria using:

dFz_net/dz < 0

  Calculates local spring constants:

- kx for transverse motion

- kz for vertical motion

  Checks that both spring constants are positive.

  Calculates trap frequencies:

- fx

- fz

  Calculates expected thermal RMS displacement in both directions.

  Calculates gas damping using:

- Stokes drag

- Cunningham-corrected Stokes drag

- Epstein drag

- Automatic drag-model choice based on Knudsen number

  Defines initial particle displacement from equilibrium.

  Runs a 2D BAOAB Langevin simulation with:

- Brownian motion

- Drag

- Optical forces

- Photophoretic force

- Gravity

- Stepwise laser-power noise

  Runs two comparable BAOAB trajectories:

- Constant laser power

- Laser power with stepwise noise

  Uses the same Brownian random kicks in both trajectories.

  Calculates the isolated laser-noise effect by subtracting:

laser-noise trajectory - constant-power trajectory

  Prints diagnostics including:

- Runtime

- Brownian RMS displacement

- Isolated laser-noise RMS effect

- Maximum laser-noise displacement effect

  Plots:

- BAOAB x(t) and z(t) motion

- Stepwise laser-power noise

- Instantaneous equilibrium shift caused by laser-power changes

- Motion with and without laser-power noise

- Isolated displacement effect from laser-power noise

- 2D trajectory in the x-z plane

- Transverse restoring-force check

- Vertical force breakdown

- 2D force-field map over beam intensity

  Replaces Fourier amplitude plots with PSD plots.

  Computes raw/non-Welch PSDs using a full-record periodogram.

  Computes Welch-averaged PSDs using scipy.signal.welch.

  Plots PSDs for:

- BAOAB x motion

- BAOAB z motion

- Isolated laser-noise displacement effect

Project_2d_cython.py

BAOAB_2d_cython.pyx

Setup_2d_cython.py

Same as Project_2d_noise.py but with cython and force lookup tables to make it run faster.

Project_3d_cython.py

BAOAB_3d_cython.pyx

Setup_3d_cython.py

  Imports tools for numerical modelling, plotting, interpolation, root finding, PSD analysis, timing, and optional Cython acceleration.

  Simulates 3D optical levitation of a particle using an Ashkin-style ray-optics force model.

  Tries to import a compiled Cython BAOAB solver:

baoab_3d_cython

and falls back to Python if it is unavailable.

  Defines the particle:

- Radius: 5 µm

- Density: 2200 kg/m^3

- Refractive index: 1.46

- Surrounding medium refractive index: 1.00027

  Defines gas properties:

- Pressure: 100 Pa

- Temperature: 300 K

- Dynamic viscosity

- Molar mass

- Molecular diameter

  Defines laser/beam properties:

- Beam waist: 6 µm

- Wavelength: 532 nm

- Beam quality factor M2 = 1.2

- Laser power: 0.041 W

  Optionally calculates the Rayleigh range from the M2 beam-quality relation.

  Adds optional stepwise laser-power noise:

- Laser noise is on

- Noise amplitude: ±1%

- Noise frequency: 300 Hz

- Step duration: 1 / 300 s

  Uses an Ashkin-style ray-optics force model.

  Samples the projected disk of the sphere using many ray bundles.

  For each ray bundle, calculates:

- Incident angle

- Refracted angle

- Fresnel reflection coefficient

- Fresnel transmission coefficient

- Scattering-like efficiency Q_s

- Gradient-like efficiency Q_g

  Computes ray-optics force components:

- Scattering-like force along the local ray direction

- Gradient-like force perpendicular to the local ray direction

  Defines photophoretic force parameters:

- Particle thermal conductivity

- Accommodation coefficient

- Thermal creep coefficient

- Absorption fraction

  Builds a cylindrical force lookup table in r-z:

- Radial grid: 101 points

- Axial grid: 201 points

- Stores radial force Fr

- Stores vertical force Fz

  Uses cylindrical symmetry to convert lookup-table forces into:

- Fx

- Fy

- Fz

  Defines equilibrium-search settings:

- Searches along the beam axis

- Finds stable equilibrium roots

- Chooses one root using equilibrium_root_index

  Calculates local trap stiffness values:

- kx

- ky

- kz

  Calculates trap frequencies:

- fx

- fy

- fz

  Calculates expected thermal RMS displacement in each direction.

  Calculates gas damping using:

- Stokes drag

- Cunningham-corrected Stokes drag

- Epstein drag

- Automatic drag-model choice based on Knudsen number

  Defines initial conditions:

- Starts at the selected equilibrium

- Initial velocity is zero in all directions

  Defines BAOAB simulation settings:

- Total time: 20 s

- Timestep: 1 / 200000 s

- Brownian noise seed

- Laser-noise seed

  Optionally supports PD feedback:

- Reads z

- Applies delay

- Estimates velocity

- Changes laser power

- Currently feedback is off

  Uses ambient gas temperature T directly for Brownian noise.

  Calculates:

- BAOAB damping factor

- Thermal velocity kick scale

  Runs BAOAB trajectories for:

- Constant laser power

- Laser power noise

- Laser power noise without Brownian noise

- Optional feedback trajectory

  Uses Cython acceleration where available for the BAOAB integration loop.

  Calculates isolated effects by subtracting trajectories:

- Brownian-only displacement effect

- Laser-noise-only displacement effect

- Feedback effect if enabled

  Downsamples long arrays for plotting using max_plot_points.

  Plots 3D BAOAB motion:

- x(t)

- y(t)

- z(t)

  Plots laser-power noise versus time.

  Plots instantaneous equilibrium shift caused by laser-power noise.

  Plots motion with and without laser-power noise.

  Plots isolated Brownian displacement effect.

  Computes PSDs for:

- BAOAB x motion

- BAOAB y motion

- BAOAB z motion

  Plots standard PSDs for x, y, and z.

  Repeats the PSD plots using Welch averaging:

- Welch segment duration: 2 s

- Prints Welch nperseg

- Prints Welch frequency bin size

  Optionally simulates detector position resolution:

- Quantises z position to 100 nm

- Compares true and quantised z trajectories

- Compares true and quantised z PSDs

- Also repeats this comparison with Welch averaging

  Computes PSDs for isolated laser-noise displacement effects:

- x

- y

- z

  Repeats isolated laser-noise PSDs with Welch averaging.

  Plots the 3D trajectory and 2D projections.

  Plots transverse restoring-force checks.

  Plots vertical force breakdown along the beam axis.

  Plots a 2D force-field map over beam intensity.

  The surface-temperature and PSD-temperature-recovery calculations have been removed.

Project_3d_no_cython.py

Same as project_3d_cython.py but without cython to compare and check both code produce the same results.

Project_3d_temp.py

Brownian_free_particle.py

Brownian_noise_spring.py

Brownian_solver_testing.py

Fourier_testing.py

Nyquist_limit.py

Photophoresis_testing.py

Compare_psd_position_resolution.py

Parameter_sweep_rms.py